Bonneville T120 & Bonneville T120 Black

SPECIFICATION

Engine and transmission

|  | BONNEVILLE T120 | BONNEVILLE T120 BLACK |
| --- | --- | --- |
| Type | Liquid cooled, 8 valve, SOHC, 270° crank angle parallel twin | Liquid cooled, 8 valve, SOHC, 270° crank angle parallel twin |
| Capacity | 1200cc | 1200cc |
| Bore/Stroke | 97.6mm x 80mm | 97.6mm x 80mm |
| Compression Ratio | 10.0:1 | 10.0:1 |
| Maximum Power | 80 PS/ 79 Bhp (59 kW) @ 6550 rpm | 80 PS/ 79 Bhp (59 kW) @ 6550 rpm |
| Maximum Torque | 105 Nm @ 3100 rpm | 105 Nm @ 3100 rpm |
| Fuel system | Multipoint sequential electronic fuel injection | Multipoint sequential electronic fuel injection |
| Exhaust | Chromed 2 into 2 exhaust system with twin chrome silencers | Black 2 into 2 exhaust system with twin black silencers |
| Final drive | X ring chain | X ring chain |
| Clutch | Wet, multi-plate assist clutch | Wet, multi-plate assist clutch |
| Gearbox | 6-speed | 6-speed |

Fuel consumption and emissions

| Standard | EURO 4 |
| --- | --- |
| Fuel Consumption | 4.5 l/100km (63 MPG) |
| Emissions | CO2 - 103.0 g/km |

CO2 and fuel consumption are measured according to regulation 168/2013/EC.  Figures on fuel consumption are derived from specific test conditions and are for comparative purposes only. They may not reflect real driving results.

Chassis

|  | BONNEVILLE T120 | BONNEVILLE T120 BLACK |
| --- | --- | --- |
| Frame | Tubular steel cradle | Tubular steel cradle |
| Swingarm | Twin-sided, tubular steel | Twin-sided, tubular steel |
| Front Wheel | 32-spoke 18 x 2.75in | 32-spoke 18 x 2.75in |
| Rear Wheel | 32-spoke 17 x 4.25in | 32-spoke 17 x 4.25in |
| Front Tyre | 100/90-18 | 100/90-18 |
| Rear Tyre | 150/70 R17 | 150/70 R17 |
| Front Suspension | Kayaba 41mm cartridge forks, 120mm travel | Kayaba 41mm cartridge forks, 120mm travel |
| Rear Suspension | Kayaba twin shocks with adjustable preload, 120mm rear wheel travel | Kayaba twin shocks with adjustable preload, 120mm rear wheel travel |
| Front Brake | Twin 310mm discs, Nissin 2-piston floating calipers, ABS | Twin 310mm discs, Nissin 2-piston floating calipers, ABS |
| Rear Brake | Single 255mm disc, Nissin 2-piston floating caliper, ABS | Single 255mm disc, Nissin 2-piston floating caliper, ABS |

Dimensions and weight

|  | BONNEVILLE T120 | BONNEVILLE T120 BLACK |
| --- | --- | --- |
| Length | 2170 mm | 2170 mm |
| Width (Handlebars) | 785 mm | 785 mm |
| Height (Without Mirrors) | 1125 mm | 1125 mm |
| Seat Height | 785 mm | 785 mm |
| Wheelbase | 1445 mm | 1445 mm |
| Rake | 25.5º | 25.5º |
| Trail | 105.2 mm | 105.2 mm |
| Dry Weight | 224 Kg | 224 Kg |
| Fuel Tank Capacity | 14.5 L | 14.5 L |

Standard equipment

ABS Traction Control Ride-by-Wire Riding Modes LED DRL headlight Heated grips Centre stand Passenger grab rail Immobiliser LED rear light